Combined Series and Parallel Lab

Instructions: Construct each of the circuits below using the PhET Circuit Simulation.  Each light bulb/resistor is 10 Ω by default.  The battery has a potential difference of 9 V by default. Complete the RVIP charts mathematically and check your answers with the “Non-Contact Ammeter” and “Voltmeter.” Then answer the questions following each diagram.

R2

S3

S2

S1

R3

R1

Series Circuit in a Parallel Circuit

|  | R | V | I | P |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 23 |  |  |  |  |
| battery |  |  |  |  |

- On a separate sheet of paper, draw a schematic version of this circuit ( and ) and then draw simplified versions to solve.

- Explain which part of the circuit is in series.  Explain which part of the circuit is in parallel.

- Compare the current in the top branch to the current in the middle branch.   Explain why.

- Rank the light bulbs in order of brightness. In terms of current flow and resistance, explain why.

- If bulb R2 were removed (right-click to remove), explain what happens to the other two bulbs and why.

- Determine which bulbs are affected by each of the switches (S1, S2, S3).  Explain why.

R3

R2

R1

Parallel Circuit in a Series Circuit

|  | R | V | I | P |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 12 |  |  |  |  |
| battery |  |  |  |  |

- On a separate sheet of paper, draw a schematic version of this circuit ( and ) and then draw simplified versions to solve.

- Explain which part of the circuit is in series.  Explain which part of the circuit is in parallel.

- Rank the light bulbs in order of brightness. In terms of current flow and resistance, explain why.

- Compare the potential difference across R3 to the potential difference across the other two bulbs. Explain why.

- If R3 were removed (right-click to remove), explain what happens to the other two bulbs and why.

- If R2 were removed, what kind of circuit does this become?

- After R2 is removed, determine what happens to the brightness of each bulb and explain why. (Hint: Complete the chart if you get stuck.

|  | R | V | I | P |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 3 |  |  |  |  |
| battery |  |  |  |  |

R4

R3

R2

R1

Series Circuit in a Parallel Circuit in a Series Circuit (Bonus)

|  | R | V | I | P |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 23 |  |  |  |  |
| 123 |  |  |  |  |
| battery |  |  |  |  |

- On a separate sheet of paper, draw a schematic version of this circuit ( and ) and then draw simplified versions to solve.

- Explain which parts of the circuit are in series.  Explain which part of the circuit is in parallel.

- Rank the light bulbs in order of brightness. In terms of current flow and resistance, explain why.