Series and Parallel Circuits

Online Lab

Open the circuit constructor at the following URL:

http://phet.colorado.edu/sims/circuit-construction-kit/circuit-construction-kit-dc_en.jnlp

Create the circuit shown in figure 1.

Make the following modifications to the component properties by right-clicking on each component:

- Set the resistance of each light bulb to 5 ohms

- Set the battery voltage to 10 volts

Figure 1

- What is the total resistance of these series-connected bulbs?

| R = |  |
| --- | --- |

- Use ohms law to calculate the circuit current (I):

Calculated current I = V/R where:

V=voltage in volts

R=Resistance in ohms

I = Current in amps (The letter “I” originally stood for Intensity)

| I = |  |
| --- | --- |

- Use the Non-Contact Ammeter to measure the current the circuit. Record the measured current.

| I = |  |
| --- | --- |

The bulbs in figure 1 are connected in series.

Connect another bulb in series with the first two. Change the properties of this bulb to have 5 ohms of resistance.

- Calculated the circuit current for the modified circuit and record this calculated current.

| I = |  |
| --- | --- |

- Use the Non-Contact Ammeter to measure the current in the circuit. Record the measured current.

| I = |  |
| --- | --- |

Connect the bulbs in the configuration shown below. Make sure each bulb is set for 5 ohms of resistance and the battery is set for 10 volts.

- What do you immediately notice about the bulbs and why?

Figure 2

The bulbs in figure 3 are connected parallel.

Figure 3

- What is the total resistance of these parallel-connected bulbs?

| R = |  |
| --- | --- |

- Calculated the currents for the parallel circuit and record these calculated values.

| I (point 1) = |  |
| --- | --- |
| I (point 2) = |  |
| I (point 3) = |  |
| I (point 4) = |  |
| I (point 5) = |  |
| I (point 6) = |  |

- Use the Non-Contact Ammeter to measure the currents in the circuit. Record the measured currents.

| I (point 1) = |  |
| --- | --- |
| I (point 2) = |  |
| I (point 3) = |  |
| I (point 4) = |  |
| I (point 5) = |  |
| I (point 6) = |  |

If the calculated values of current don’t match the measured values of current read about parallel circuits in your text and recalculate the currents.

- As you add more bulbs (loads) in a series circuit what happens to the current in the series circuit and why?

- As you add more bulbs (loads) in a parallel circuit what happens to the current in the series circuit and why?

- Which one of these circuits represents the type of electrical connections found in your home?

Series (figure 4) and parallel (figure 5) connected loads.

Figure 5

Figure 4

Just as loads like the light bulbs can be connected in series and parallel the power sources can also be connected in series and parallel.

- What is the advantage of connecting power supplies in series?

- What is the advantage of connecting power supplies in parallel?