Name:

Class:

HSA Brownsville Science Department

Circuits Virtual Lab

Fatih Gozuacik, M.Ed, M.Phys

Goal of the experiment: To understand & explore circuit elements, equivalent resistance, current and voltage calculations.

Needed materials:  A laptop or desktop with Java software, PhEt Circuits Lab simulation, and a calculator.

Download the simulation: https://phet.colorado.edu/en/simulation/legacy/circuit-construction-kit-ac

Step 1: Series Circuit. Create a series circuit by pulling resistors from the side, and then right click on the resistor.

| Change the resistance. | Pick any three different value of your choice. And set the voltage by right clicking to any value of desire. Build a circuit as in above, you can use your imagination and alter the diagram. You can replace one resistor with a light bulb and change the resistance. A switch can also be added. |  |
| --- | --- | --- |

|  |  |  |  |  |  | Theoretical Value |  |  |  | Experimental Value |  |  |  | Pull a volt meter from the side to check individual voltages. Connect ampere meter to measure current. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | R1 | R2 | R3 | Req | V | I | V1 | V2 | V3 | I | V1 | V2 | V3 |  |
| Set 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Set 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Step 2: Parallel circuit. Use above steps to build a three resistor system of any choice and desired voltage.

|  | Build a circuit similar to the one left. Change resistors and voltage and calculate theoretical values. Then use a voltmeter and ampere meter to have experimental values to compare. A sample set up can be seen on the right. |  |
| --- | --- | --- |

|  |  |  |  |  |  | Theoretical Value |  |  |  |  |  |  | Experimental Value |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | R1 | R2 | R3 | Req | V | Im | I1 | I2 | I3 | V1 | V2 | V3 | Im | I1 | I2 | I3 | V1 | V2 | V3 |
| Set 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Set 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Use back side for result and commentary

Step 3: Result and commentary:

Explain the experiment step by step what you did. Did your theoretical and experimental calculations match?

If not, explain why.  Add improvements if any.

Question 1: Is the equivalent resistance always lower than any resistor in parallel connection?

Question 2: Which connection should be chosen for Christmas lights? Series or parallel? Why?

Question 3: What type of connections do we use in our homes, offices and daily life? Why?

Question 4: Build a short circuit and explain what happens.