Name, Date___________________________

Series and Parallel Circuits

This activity is designed to be run with the PhET circuit construction kit, (NOT the “virtual lab” version) available online. The ۞ symbol indicates you should check in with an instructor.

- Simple Circuit- One Resistor

  - ۞ Construct a simple circuit that contains the following components:  Light bulb, 10-V battery, and an ammeter.

    - Make sure that the components are in series; that is, you can trace a charge traveling through the entire circuit from the positive end of the battery to the negative end.

    - Right-click the battery and the light bulb, and check the “Show Value” box.

    - Call your instructor to check your setup.

  - Use the voltmeter to confirm that the voltage of the battery is, in fact, 10 volts.  If it is not 10 volts, change it by right-clicking the battery and using the keyboard to enter in 10V.

  - Record the current flowing through the circuit by reading the ammeter value:

Itotal = _________

  - **Use V=IR to predict the resistance of the light bulb, and record the answer below.

Rbulb = __________

  - Confirm this resistance by right-clicking on the light bulb and then clicking the “Change Resistance” button.  You should see the resistance listed on the screen.

  - ۞  ** Was the equation V = IR verified here?   Why or why not?

- Simple Series Circuit- Two Resistors

  - Add another light bulb in series to the simple circuit above.  Right-click the light bulb to make sure it is also a 10-Ω light bulb.

  - Confirm that the battery is still producing a 10-volt potential difference using the voltmeter.

  - Determine the current flowing through the circuit by recording the ammeter reading below:

Itotal =  __________

  - **Using V=IR, calculate the total resistance of the circuit, and record this value below.  Show your work.

Rtotal = _____________

  - **Add a third light bulb to the circuit by connecting it in series to the others.  Repeat steps bd and calculate the total resistance of the 3 bulbs, and record this resistance.  This is called the equivalent resistance of the circuit.

Req = ______________

  - **You should notice a trend developing for resistors that are connected in series.  Based on this trend, predict the equivalent resistance of a series of 5 light bulbs, 4 with a resistance of 10 Ω and the 5th with a resistance of 20 Ω.  Confirm your prediction by building this circuit.

Answer:  Req = ____________

  - ۞ Now, for a final definition of equivalent resistance:   given a series of resistors R1, R2, R3, R4…etc.,  write a formula that determines the total equivalent resistance Req.  Check in with instructor.

- Simple Series Circuit:  Voltages & Currents of Components

  - Next, return to a 3-bulb setup and change the resistances of the light bulbs until you have:  one 10-Ω bulb, one 20-Ω bulb, and one 30-Ω bulb, all connected in series.  Also, remove the ammeter.

  - Using the rule developed from part 2(f) above, calculate the equivalent resistance of the 3 bulbs, and record this resistance

here: Req = ________________

  - Use the voltmeter to determine the voltages across each of the three light bulbs.  Record these voltages below in the appropriate spots.

V10Ω = _________	  V20Ω = _________ 	     V30Ω = _________

  - **How are these voltages related to the total voltage across the battery?

  - ۞ ** What can be said, in general, about the relationship between the voltages of components that are connected in series, and the total voltage across the terminals of the battery?

  - **Predict the current flowing through the circuit by using V=IR.  Use Req for the circuit as your R, and use the voltage of the battery for V.  Record your current  prediction below:

I = ____________

  - Put the ammeter back into the circuit, and check your prediction.

  - Use the “non-contact ammeter” from the menu to check the current at various places around the circuit.  Are there any changes?

  - What can be said, in general, about the current values at different places within a series circuit?

- Simple Parallel Circuit:  Two Resistors

  - ۞ Clear the screen and set up a simple circuit identical to the setup from part 1.  Make sure the battery is a 10-Volt battery.  Then, connect a second 10-Ω light bulb in parallel to the first bulb.  This means that the circuit splits for the 2 bulbs.  Check in with instructor.

  - **Use the voltmeter to measure the voltages across each bulb.  How do these 2 voltages compare to each other?  How do they compare to the voltage across the battery terminals?

  - **Increase the resistance of one bulb to 100 Ω, then repeat step 4(b).  Did the voltages change significantly?  How?

  - **What can be said, in general, about the voltages for components that are connected in parallel to each other?

  - Reset both resistances to 10 Ω.   Using the ammeter reading, and the known voltage of the battery, use V=IR to calculate the equivalent resistance of the circuit.

Req = ____________

  - Change one bulb from 10 Ω to 20 Ω, and repeat step 4(e).

Req = ____________

  - Change the 20 Ω bulb to 30 Ω, and repeat 4(e).

Req = ____________

  - Try to determine the pattern developing.   How is the equivalent resistance related to the values of the resistors used in a parallel circuit?  Ask your instructor or look in your textbook for the correct formula for parallel circuits.

  - ۞ Using the formula given to you by your instructor, predict the equivalent resistance of 3 10-Ω bulbs that are connected in a parallel circuit.   Check with instructor.

  - After showing your prediction to your instructor, confirm your prediction by setting up a 3-bulb circuit on the computer and using the same technique from 4(e).

- Simple Parallel Circuit:  Voltages & Current of Components

  - Construct a 3-bulb parallel circuit, and change the resistances of the light bulbs until you have:  one 10-Ω bulb, one 20-Ω bulb, and one 30-Ω bulb.

  - Using the rule developed above, calculate the equivalent resistance of the 3 bulbs in parallel, and record this resistance

here: Req = ________________

  - Use the voltmeter to determine the voltages across each of the three light bulbs.  Record these voltages below in the appropriate spots.

V10Ω = _________	  V20Ω = _________ 	     V30Ω = _________

  - **How are these voltages related to the total voltage across the battery?

  - ۞ ** What can be said, in general, about the relationship between the voltages of components that are connected in parallel, and the total voltage across the battery?

  - Knowing the equivalent resistance and the battery voltage, calculate the total current in the circuit using V=IR. Write your answer below, showing your work.

Total Current = ______________

  - Drag the “non-contact ammeter” from the menu, and place it on one of the wires directly connected to the battery.  Confirm that the current is what you predicted.

  - Knowing the voltage across each resistor, you can calculate the current for each one.  Do so, and write your answers below. Show your work.

I10Ω = _________	  I20Ω = _________ 	     I30Ω = _________

  - Place the non-contact ammeter on each of the three wires connected to a single bulb, and check your answers for each current calculation.

  - ۞How do these individual currents compare to the total current in the circuit? Why does this make sense?

- Practice Problems

  - Series Circuits

    - Sketch a series circuit that contains the following:  A 9.0-V battery, 2 20-Ω resistors, and an Ammeter.

    - What is the equivalent resistance of this circuit? (show work)

    - What reading would the ammeter show?

    - What is the voltage across each resistor?

  - Parallel Circuits

    - Sketch a parallel circuit that contains the following:  a 9.0-V battery, 3 15-Ω light bulbs in parallel.

    - Calculate Req for this circuit.

    - Calculate the current flowing through one of the bulbs.