TFCPHYSICS 91F

/ 9

Lab 4

Lab 4 - Hooke's Law

Part 1: Validating Hooke’s Law

| Mass added (g) | Weight (N) | Extension (mm) |
| --- | --- | --- |
| 100g | 0.98 | 18 |
| 150g | 1.47 | 26 |
| 200g | 1.96 | 34 |
| 250g | 2.45 | 42 |
| 300g | 2.94 | 50 |

Table 1

f)the spring constant =  0.0612Nmm-1

g) applied force v/s extension is  a straight line

Part 2: Determining Unknown Masses

- Following the steps in part 1, place the red mass on the spring and determine the extension .

- Repeat the procedure 3 times and list the values as shown in the box below.

- Follow the same procedure for blue mass as well

| Mass added (g) | Extension (mm) |
| --- | --- |
| red | 62 |
|  | 62 |
|  | 62 |
| blue | 39 |
|  | 39 |
|  | 39 |

- Using the value of k from part 1 in the equation ,

F = k x

k= 0.0612Nmm-1

- For red mass,                                        m * g = 0.0612 * 62

m =     (0.0612 * 62) / 9.8

red mass = 387.183g

- For red mass,                                        m * g = 0.0612 * 39

m =     (0.0612 * 39) / 9.8

blue mass =243.551g

Part 3: Determining Unknown Value of Gravity

| Mass added (g) | Extension (mm) | Unknown g |
| --- | --- | --- |
| 300g | 72 | 14.68 |
| 387.183g | 89 | 14.067 |
| 243.551g | 56 | 14.07 |

F = k x

k= 0.0612Nmm-1

unknown g =14.272 ms-2

Part 4: Analysing Systems of Springs

Two Springs in Parallel

| k1 (Nm-1) | k2 (Nm-1) | F (N) | Extension (m) | Total k (Nm-1) |
| --- | --- | --- | --- | --- |
| 200 | 200 | 100 N | 0.250 | 400 |
| 200 | 400 | 100 N | 0.167 | 598.802 |
| 400 | 600 | 100 N | 0.100 | 1000 |
| 200 | 600 | 100 N | 0.125 | 800 |

Table 2

d) F = kx

Ftotal = k1 x + k2 x

= (k1 + k2)x

Two Springs in Series

| k1 (Nm-1) | k2 (Nm-1) | F (N) | Extension (m) | Total k (Nm-1) |
| --- | --- | --- | --- | --- |
| 400 | 400 | 100 N | 0.500 | 200.00 |
| 200 | 400 | 100 N | 0.750 | 133.33 |
| 600 | 400 | 100 N | 0.417 | 239.81 |
| 200 | 600 | 100 N | 0.667 | 149.93 |

Table 3

h)

F = kx

Ftotal =(k1k2)/(k1+k2) x

Student 3 has correct model